Sparwix inter.

Proposed 3BHK Floor Plan (1300 sq.ft, 50×26 ft plot)

We consider a rectangular plot 50 ft × 26 ft (1300 sq.ft), which yields an external wall perimeter of 152 ft (2×(50+26)). The entire built-up area is capped at 1300 sq.ft. The design allocates space for 3 bedrooms, a combined living/dining hall, kitchen, two bathrooms (one attached to the master bedroom and one common), and a dedicated car parking at the front. Key room dimensions (in feet) are chosen to match Indian norms (e.g. living ~14×16 to 18×24 ft, master bedroom ~12×12 ft, other bedrooms ~10×10 ft). The layout places the car porch (9×15 ft) at the front (west side) with a 10 ft main gate (MG) for vehicle access, and the hall and kitchen adjacent. One typical 3BHK layout for a similar plot (27×50 ft, 1350 sq.ft.) is shown below﻿
Illustrative example of a 3BHK layout (ground floor) on a ~27×50 ft plan. Our design uses a similar arrangement of hall, bedrooms, kitchen, bathrooms and parking consistent with Indian standards.﻿
The proposed plan (text diagram below) organizes the spaces as follows: the living hall (with dining) opens from the entrance and extends roughly 22×14 ft (≈308 sq.ft) – within the recommended 14×16 to 18×24 ft range. The kitchen is placed adjacent (about 9×14 ft), and one bedroom (10×10 ft) is located at the front-right corner. The two remaining bedrooms lie towards the rear: a Master Bedroom (~12×12 ft) with an attached bath (5×8 ft), and Bedroom 2 (~10×12 ft) also with a bath (5×8 ft). The car parking porch (9×15 ft) sits under the front-left, leading directly to the main gate. All room dimensions and areas are labeled in the plan below. Doors (D) of ~3 ft width are shown at each room entry, and windows (W) of ~4–5 ft width are placed on exterior walls for daylight and ventilation. All walls are drawn with realistic thickness (9″ external, 4.5″ internal – typical brick dimensions).﻿
50 ft +------------------------------------------------------------+ | [MG 10'-0"] Living Hall/Dining (22'×14', 308 sq.ft) | | +------------------------------------+ | | Car 9×15 | | Bed3| | Park | |10×10| | (9′×15′) | |ft | | | |guest| | | |room | | +------------------------------------+ | +------------------------------------------------------------+ | Master Bedroom 12×12 (144 sq.ft) | Bedroom2 10×12 (120 sq.ft) | Kitchen 9×14 (126 sq.ft) | | (with attached 5×8 bath) | (w/ attached 5×8 bath) | (incl. window) | +------------------------------------------------------------+ (26 ft total depth)﻿
Dimensions & Area Summary: The table below lists each space with its length×width and area. These comply with practical Indian norms (e.g. a living hall ~14–18 ft wide, bedroom 10–12 ft wide, etc.).﻿

| Space | Dimensions (L × W, ft) | Area (sq.ft) |
| --- | --- | --- |
| Living Hall (+ Dining) | 22 × 14 | 308 |
| Kitchen | 9 × 14 | 126 |
| Master Bedroom | 12 × 12 | 144 |
| Master Bath (attached) | 5 × 8 | 40 |
| Bedroom 2 | 10 × 12 | 120 |
| Bedroom 2 Bath | 5 × 8 | 40 |
| Bedroom 3 (Guest) | 10 × 10 | 100 |
| Car Parking (Porch) | 9 × 15 | 135 |
| Total (rooms only) |  | 1013 |

Annotations/Notes:

- Wall Thickness: External walls are ~9 in (230 mm) thick and internal partitions ~4.5 in (115 mm).

- Doors: Standard door frames ~7×3 ft (84″×36″). Main entrance gate is ~10 ft wide.

- Windows: Typical window openings are about 4–5 ft wide (for light/ventilation). Each exterior room has at least one window on a suitable wall.

- Car Parking: A space ~9×15 ft accommodates a sedan (the norm is ~8×16 ft). We provide a 9×15 porch under a roof (open on one side) accessed via the 10 ft gate.

- Circulation: A short passage connects bedrooms and bathrooms. All rooms are directly accessible from the hall or corridor, ensuring privacy and convenience.

This layout efficiently uses the 1300 sq.ft plot while reflecting practical Indian 3BHK design (ample hall, attached toilets for two bedrooms, and dedicated parking).﻿
Sources: Design standards and room-size guidelines are drawn from Indian residential planning references, ensuring the scheme is realistic and up-to-date.﻿
50 ft﻿
+------------------------------------------------------------+﻿
|  [MG 10'-0"]      Living Hall/Dining (22'×14', 308 sq.ft)   |﻿
|                 +------------------------------------+     |﻿
|  Car 9×15      |                                    | Bed3|﻿
|  Park          |                                    |10×10|﻿
|  (9′×15′)      |                                    |ft   |﻿
|                |                                    |guest|﻿
|                |                                    |room |﻿
|                +------------------------------------+     |﻿
+------------------------------------------------------------+﻿
| Master Bedroom 12×12 (144 sq.ft) |  Bedroom2 10×12 (120 sq.ft) | Kitchen 9×14 (126 sq.ft) |﻿
| (with attached 5×8 bath)         | (w/ attached 5×8 bath)       | (incl. window)           |﻿
+------------------------------------------------------------+﻿
(26 ft total depth)﻿

﻿